Preview Hospital

Laboratory and Imaging Safety

(Mandatory System Documentation as per NABH SHCO 3rd Edition — AAC.6.a, AAC.6.b, AAC.6.c, AAC.6.d, AAC.6.e)

| Document No. | PREVIEW/AAC/POL-6 | Version | 1.0 |
| --- | --- | --- | --- |
| Effective Date | [DD-MM-YYYY] | Review Date | [DD-MM-YYYY] |
| Applicable OE(s) | AAC.6.a, AAC.6.b, AAC.6.c, AAC.6.d, AAC.6.e | NABH Chapter | AAC |
| Prepared By | _________________ | Date | ________ |
| Reviewed By | _________________ | Date | ________ |
| Approved By | _________________ | Date | ________ |

# OE Cross-Reference

| OE Code | Requirement | Addressed In | Responsible |
| --- | --- | --- | --- |
| AAC.6.a | The laboratory-safety programme is implemented | Steps 1, 2, 3, 8 | The named laboratory-safety lead writes, walks and revises the programme and records incidents; laboratory personnel work inside it and report incidents; infection-prevention owns standard precautions and bloodborne-pathogen sharps including in specimen-handling areas; environment-and-waste owns the biomedical-waste stream and the State Pollution Control Board authorisation; facility-management fire plans receive laboratory fire events; quality or accreditation coordinator audits implementation |
| AAC.6.b | Laboratory personnel are appropriately trained in safe practices and are provided with appropriate safety measures | Steps 4, 8 | The named laboratory-safety lead trains and records competence; administration provides the measures at the bench; quality or accreditation coordinator audits |
| AAC.6.c | Patients are appropriately screened for safety / risk before imaging | Steps 5, 8 | The named screening role completes the checklist before the study; imaging personnel do not begin an unscreened study; quality or accreditation coordinator audits |
| AAC.6.d | Imaging personnel and patients use appropriate radiation safety and monitoring devices where applicable, and are trained in imaging safety practices and radiation-safety measures | Steps 6, 8 | The person who issues monitoring devices keeps the records; imaging personnel wear them and use intact protective devices; quality or accreditation coordinator audits |
| AAC.6.e | Imaging signage is prominently displayed in all appropriate locations | Steps 7, 8 | The named signage role keeps signs visible; imaging-services policy owns the licences the signs warn about; definition-and-display policy owns service boards; infection-control policies own isolation cards; quality or accreditation coordinator audits |

# Abbreviations

| Abbreviation | Meaning |
| --- | --- |
| AERB | Atomic Energy Regulatory Board |
| ALARA | As Low As Reasonably Achievable |
| OSL | Optically Stimulated Luminescence (dosimeter) |
| PC-PNDT | Pre-Conception and Pre-Natal Diagnostic Techniques (Prohibition of Sex Selection) Act |
| TLD | Thermoluminescent Dosimeter |
| UID | Unique Identification Number |

Abbreviations already defined in the HIC.1 to HIC.6 master policies are not repeated here. A reader using this document on its own should refer to those policies for the shared glossary, including NABH, SHCO, OE, BMW and PPE.

The following abbreviations are used in this document and are not defined in HIC.1 to HIC.6:

Any additional abbreviation used locally within Preview Hospital is [Hospital to define] and is added to this list at the next revision.

# 1. Purpose

This document sets out how Preview Hospital keeps the laboratory and the imaging service safe for the people who work in them and for the patients who pass through them: a laboratory-safety programme that is actually implemented; laboratory personnel trained and equipped to work safely; patients screened for safety and risk before imaging; radiation-safety devices and monitoring where they apply; and imaging signage that is up, in the right places, and saying the right thing.
The chapter intent is that laboratory and imaging services are provided by competent staff in a safe environment for both patients and staff. The laboratory-services policy and the imaging-services policy of Preview Hospital own the work those services do. This document owns that the environment in which that work is done is safe.
A safety programme that exists as a folder, or signage that exists as a purchase order, is not a safe environment. This document is the programme that has to be running on the day a reagent spills, a sharp breaks, a patient of childbearing age is about to be irradiated, or a visitor walks toward a restricted door.

# 2. Scope

This policy applies to the laboratory of Preview Hospital and to every imaging modality the hospital actually provides, including any outsourced arrangement only insofar as this hospital's own staff or patients are exposed on these premises. It binds laboratory personnel, imaging personnel, the person who leads the laboratory-safety programme, the person who issues and collects personnel-monitoring devices, and whoever maintains imaging signage. It does not assume that Preview Hospital provides magnetic resonance imaging, computed tomography, interventional radiology or nuclear medicine: those modalities appear only where the defined scope includes them.
It covers the laboratory-safety programme and its implementation, including incidents and review; training of laboratory personnel and the safety measures provided to them; screening of patients for safety and risk before imaging; use of radiation-safety and monitoring devices by imaging personnel and patients, and training in imaging and radiation safety; and prominent display of imaging signage in all appropriate locations.
Boundaries with other policies of Preview Hospital:
- Infection-prevention practices — hand hygiene, standard precautions, transmission-based-precaution personal protective equipment, and bloodborne-pathogen sharps safety, including in laboratory specimen-handling areas — are governed by the infection-prevention-and-control-practices policy of Preview Hospital. This policy owns the laboratory-safety programme as a whole: biosafety cabinets where the work requires them, chemical hygiene, fire as it arises from laboratory work, reagent and specimen spills other than the blood-and-body-fluid spill already governed there, and laboratory-specific protective equipment beyond standard precautions.
- Biomedical-waste colour code, on-site transport, storage, handover to the common treatment facility, and State Pollution Control Board authorisation are governed by the environment-and-waste policy of Preview Hospital. Pre-treatment of microbiological waste before it enters that stream is named here as a laboratory-safety duty; the stream itself is not restated.
- Specimen requisition, collection, identification, handling, transportation, processing and disposal, as a pathway that produces a valid result, are governed by the laboratory-services policy of Preview Hospital. This policy owns that those practices are safe for the staff who perform them.
- Imaging licences and authorisations — AERB authorisation for diagnostic radiology, PC-PNDT registration where ultrasonography capable of sex determination is performed — are governed by the imaging-services policy of Preview Hospital. That policy owns the licences. This policy owns the safety signage, including the radiation trefoil, restricted-area notices, pregnancy caution, and the PC-PNDT notice where ultrasonography is performed.
- The public display of defined healthcare services is governed by the definition-and-display policy of Preview Hospital. Isolation and infection-control signage is governed by the infection-control policies. Neither is imaging signage.
- The hospital fire-and-non-fire emergency plans are governed by the facility-management policies of Preview Hospital. This policy owns laboratory-specific fire risk (flammable reagents, electrical equipment at wet benches) and points those events at the hospital plan; it does not rewrite that plan.
- The hospital-wide hazardous-materials programme, where one exists under facility management, owns building-level chemical stores. This policy owns chemical hygiene at the laboratory bench.

# 3. Policy Statement

Preview Hospital implements a laboratory-safety programme. The programme is written, led, used on working days, and reviewed when something goes wrong and when the calendar says so.
Laboratory personnel are trained in safe practices and are provided with the safety measures the work requires. Training that is attendance without a practice, or equipment that is locked away from the bench, is not this requirement met.
Patients are screened for safety and risk before imaging, against a written checklist appropriate to the modality actually being used. A modality this hospital does not provide is not screened for.
Where radiation is used, imaging personnel and patients use the monitoring and protective devices that apply, and are trained in imaging-safety practices and radiation-safety measures. Doses to personnel are kept as low as reasonably achievable, within the limits the AERB authorisation requires. Those limits are not restated here as numbers.
Imaging signage is displayed prominently in every location it belongs. A sign in a drawer is not displayed.

# 4. Procedure

1. The laboratory-safety programme — what it covers and why (AAC.6.a)

Preview Hospital maintains a written laboratory-safety programme covering the laboratory as a place of work, not as a testing pathway. The testing pathway — how a specimen is requested, identified, processed and reported — is owned by the laboratory-services policy of Preview Hospital. This programme exists because a laboratory that produces correct results can still injure the people who produce them, and because an assessor asking whether a laboratory-safety programme is implemented will ask what it covers, not whether the laboratory quality-assurance file can be produced.

The programme covers, as laboratory safety:

- biosafety — the handling of potentially infectious material at the bench, the use of a biosafety cabinet where the work generates infectious aerosols or where microbiological culture is performed, and the rule that mouth pipetting is not used;

- chemical hygiene — labelling of reagents, storage that separates incompatibles, access to the safety information for each reagent in use, and the prohibition on storing food in a reagent refrigerator;

- fire as it arises from laboratory work — flammable reagents, open flames where they are still used, and electrical equipment at wet benches — pointing every fire event at the hospital fire plan rather than inventing a second one;

- spills of specimens and of reagents at the bench, other than the blood-and-body-fluid spill already governed by the infection-prevention-and-control-practices policy of Preview Hospital;

- sharps generated at the laboratory bench (broken glass, coverslips, blades used in the laboratory). Sharps arising from bloodborne-pathogen exposure during specimen collection are governed by the infection-prevention policy; they are not rewritten here;

- pre-treatment of microbiological waste, where the laboratory generates it, before that waste enters the biomedical-waste stream owned by the environment-and-waste policy of Preview Hospital. Colour code, transport and the State Pollution Control Board authorisation stay with that policy.

The programme is the whole of those elements operating together. A laboratory that has a spill kit and no chemical hygiene, or a biosafety cabinet that is never certified, has a collection of objects and does not have a programme. That is why this step is the documented-evidence anchor, and why the common error is to point at the infection-control manual or the waste authorisation and call it laboratory safety.

The written programme is held at [Hospital to define — where the laboratory-safety programme is held], and the named lead is [Hospital to define — the named lead of the laboratory-safety programme].

2. Implementing the laboratory-safety programme (AAC.6.a)

Implementation is the programme in use on a working day, not the existence of the file at step 1.

On every working day the laboratory of Preview Hospital:

- works only with the protective equipment the task requires, kept at the bench and in the sizes people actually wear — laboratory-specific equipment beyond standard precautions (for example a chemical-resistant glove or an eye shield for a reagent that standard-precaution gloves will not handle) is provided under this programme; standard-precaution gloves, gowns and masks remain the infection-prevention policy's;

- uses a biosafety cabinet, where the work at step 1 requires one, that has been certified as performing and has not been treated as a storage shelf;

- stores and labels reagents so that a person who did not put the bottle down can still read what it is;

- keeps a spill kit for reagent and specimen spills at the bench, used under the written method in the programme, with blood-and-body-fluid spills continuing to follow the infection-prevention spill procedure;

- pre-treats microbiological waste, where generated, before it is placed in the yellow stream the environment-and-waste policy already defines;

- stops work that cannot be done safely — a cabinet out of certification, a reagent without a label, a spill kit that has been used and not replenished — and records the stop.

The named lead at step 1 walks the laboratory at [Hospital to define — the interval at which the laboratory-safety lead walks the laboratory] and records what was found. A walk that never finds a defect is possible; a walk that is never done is not implementation.

Outsourced laboratory work performed on another organisation's premises is not this hospital's laboratory. Outsourced work performed by visiting staff in this hospital's laboratory is.

3. Laboratory-safety incidents and review (AAC.6.a)

A laboratory-safety incident is any event the programme was built to prevent and failed to: a reagent or specimen spill, a bench-sharp injury, a chemical exposure, a fire or near-fire at the bench, work done in a biosafety cabinet that was out of certification, or microbiological waste that left the laboratory without the pre-treatment the programme requires.

Every such incident is recorded in [Hospital to define — the laboratory-safety incident register], with the date and time, what happened, who was involved, the immediate action, and the change made so that it does not recur. A bloodborne-pathogen sharps injury during specimen collection continues to be recorded and managed under the infection-prevention and occupational-health policies; it is not duplicated here. A bench-sharp injury that is not a bloodborne-pathogen exposure is recorded here and is still offered the occupational-health route if exposure cannot be excluded.

Incidents are reviewed with the named lead and at [Hospital to define — the review forum and interval for laboratory-safety incidents], and the programme at step 1 is revised when a gap is the cause. A register that is never read is not review.

The programme itself is reviewed at the same interval as this policy at step 8, and sooner after an incident that exposes a gap, after a change to the tests the laboratory performs, or after a revision of the infection-prevention or environment-and-waste policies this document hands work to.

The reason incidents sit inside the documented-evidence anchor rather than as an appendix: a programme that has never had an incident may be excellent or may be unreporting. The register, the review and the walk at step 2 are how Preview Hospital can tell which.

4. Laboratory personnel — training and safety measures (AAC.6.b)

Laboratory personnel are trained in the safe practices the programme at steps 1 to 3 requires, before unsupervised bench work, and again at [Hospital to define — the interval for laboratory-safety training]. Training covers biosafety at the bench, chemical hygiene, spill response, bench-sharps, fire as it arises in the laboratory, and the pre-treatment duty for microbiological waste. It is assessed by return demonstration of the spill method and of the protective equipment the person's work uses, not by attendance alone.

The safety measures provided — protective equipment at the bench, the spill kit, access to reagent safety information, and a biosafety cabinet where the work requires one — are provided by Preview Hospital and are not a personal purchase. A measure that is not at the point of work is not provided.

Staff who only collect specimens and do not work at the laboratory bench are trained under the infection-prevention policy for that collection work; they are trained under this step only for the laboratory tasks they actually perform.

5. Screening patients for safety and risk before imaging (AAC.6.c)

Every patient is screened for safety and risk before an imaging study is performed, against a written checklist appropriate to the modality being used. A checklist copied from a centre that has a different modality is the common defect: it asks questions this hospital cannot act on, and misses the ones it must.

The checklist is [Hospital to define — the pre-imaging safety/risk screening checklist, by modality provided]. It includes, as applicable to the modalities Preview Hospital actually provides:

- pregnancy status, or the possibility of pregnancy, before any study using ionising radiation to the abdomen or pelvis of a person who can be pregnant, and before any study the local protocol treats as requiring that question;

- previous reaction to contrast, and risk factors for contrast-associated kidney injury, where a contrast-enhanced study is intended;

- implanted or attached metal, electronic implants and other contraindications, where magnetic resonance imaging is provided.

Where a modality is not provided, it is not screened for. Magnetic resonance imaging is not assumed.

A study is not begun until the checklist for that modality is complete. A positive screen is acted on — delayed, modified, or not performed — and the action is recorded against the unique identification number. Screening is performed by [Hospital to define — who performs pre-imaging safety/risk screening].

6. Radiation-safety devices, monitoring and training (AAC.6.d)

Where ionising radiation is used, imaging personnel and patients use the radiation-safety and monitoring devices that apply, and are trained in imaging-safety practices and radiation-safety measures.

Personnel who work with radiation-generating equipment wear a personnel-monitoring device — thermoluminescent dosimeter or optically stimulated luminescence dosimeter — issued, worn at the position the AERB authorisation requires, returned at [Hospital to define — the interval at which personnel-monitoring devices are issued and returned], and not left in a drawer or shared between two people. Dose records are held as the AERB authorisation requires. Doses are kept as low as reasonably achievable and within the limits that authorisation requires. Those limits are not restated in this document as numbers.

Protective devices — lead aprons, thyroid shields, and any other device the protocol for that study requires — are available, intact, and used by personnel and, where the protocol provides, by the patient. A device with a cracked or thinned protective layer is withdrawn. Patient shielding is used where it is applicable to the study and does not obscure the region being examined.

Training in these practices is given before unsupervised work with radiation-generating equipment, and again at [Hospital to define — the interval for imaging and radiation-safety training]. Staff who do not work with radiation are not trained as if they did.

ALARA is the principle — as low as reasonably achievable — not a figure. Technique, collimation, and not repeating a study without a reason are how the principle is applied. A numeric dose constraint copied from an old circular is not this step.

7. Imaging signage (AAC.6.e)

Imaging signage is displayed prominently in every location it belongs: at the entrance to any area where radiation-generating equipment is used, on the door of a restricted area, in the waiting area from which a patient of childbearing age may be called, and, where ultrasonography capable of sex determination is performed, the statutory PC-PNDT notice.

The signs include, as applicable: the radiation trefoil; a restricted-area notice; a pregnancy-caution notice instructing a patient who is or may be pregnant to tell staff before a study using ionising radiation; and the PC-PNDT notice where ultrasonography is performed.

This signage is not the service display owned by the definition-and-display policy of Preview Hospital, and it is not isolation or infection-control signage owned by the infection-control policies. A board that lists the hospital's specialties does not satisfy this step. A precaution card on an isolation room does not satisfy this step.

The person who maintains the signs is [Hospital to define — the role that maintains imaging signage]. A missing, faded or obstructed sign is replaced. A sign stored for an inspection is not displayed.

Licences behind the activity the signs warn about remain on the imaging-licence calendar under the imaging-services policy of Preview Hospital.

8. Records, review and the order of operations (AAC.6.a, AAC.6.b, AAC.6.c, AAC.6.d, AAC.6.e)

Every laboratory-safety walk, incident, training session and pre-treatment record; every pre-imaging screening checklist; every personnel-monitoring issue and dose record; and the current imaging signage locations are recorded and retrievable. Where a patient is involved, the unique identification number issued at registration is used.

The quality or accreditation coordinator audits a sample of these records at [Hospital to define — the audit interval for laboratory-and-imaging-safety records] for a laboratory-safety programme that was walked and whose incidents were reviewed, for training that included a demonstration, for screening checklists completed before the study, for personnel-monitoring devices issued and returned, and for signage present at the locations step 7 names.

This policy is reviewed at [Hospital to define — the review interval for this policy], and sooner when the laboratory's test list or the imaging scope changes, when a laboratory-safety or radiation incident exposes a gap, or when the laboratory-services, imaging-services, infection-prevention or environment-and-waste policies this document hands work to are revised.

# 5. Responsibility

The head of the institution is accountable for Preview Hospital running a laboratory-safety programme rather than holding a file, and for radiation work being performed only with the monitoring, devices and signage the AERB authorisation and this policy require.
The named lead of the laboratory-safety programme writes and walks the programme, records incidents, sees that microbiological waste is pre-treated before it enters the biomedical-waste stream, and revises the programme when a gap is found.
Laboratory personnel follow the programme, use the safety measures provided, complete training before unsupervised bench work, and report incidents.
Imaging personnel complete the pre-imaging checklist before a study, wear and return personnel-monitoring devices, use protective devices that are intact, and do not begin a study the screen has not cleared.
The person who issues personnel-monitoring devices keeps the issue, return and dose records the AERB authorisation requires.
The role that maintains imaging signage keeps every required sign visible in its location.
The infection-prevention team continues to own hand hygiene, standard and transmission-based precautions, and bloodborne-pathogen sharps, including in specimen-handling areas. The environment-and-waste function continues to own the biomedical-waste stream and the State Pollution Control Board authorisation. This policy does not take those duties over.
The quality or accreditation coordinator audits the records at step 8 and reports findings to the head of the institution.
All staff are expected to treat an unscreened imaging study, a monitoring device left in a drawer, a missing radiation sign, or laboratory work done without the protective equipment the task requires, as a defect, and to report it.

# 6. References

- National Accreditation Board for Hospitals and Healthcare Providers (NABH), Standards for Small Healthcare Organisations, 3rd Edition — Access, Assessment and Continuity of Care chapter, standard AAC.6.
- Bio-Medical Waste Management Rules, 2016 — pre-treatment of microbiological waste before it enters the colour-coded stream; the stream, transport and State Pollution Control Board authorisation remain with the environment-and-waste policy of Preview Hospital.
- Atomic Energy Act, 1962, and the Atomic Energy (Radiation Protection) Rules, together with the authorisations issued by the Atomic Energy Regulatory Board — personnel monitoring, protective devices, restricted areas and the dose records the authorisation requires.
- Pre-Conception and Pre-Natal Diagnostic Techniques (Prohibition of Sex Selection) Act, 1994 — the statutory notice, where ultrasonography is performed. Registration under that Act is owned by the imaging-services policy of Preview Hospital.
- World Health Organization, Laboratory Biosafety Manual, 4th edition (2020) — the principle that laboratory safety is a programme covering biosafety, chemicals, fire, spills and sharps, applied here without importing that manual's facility grades as requirements.
- Internal documents of Preview Hospital: the laboratory-safety programme and incident register; laboratory-safety training records; the pre-imaging screening checklist; personnel-monitoring and dose records; the imaging-signage locations; the laboratory-services policy; the imaging-services policy (licence calendar); the infection-prevention-and-control-practices policy; the environment-and-waste policy; and the hospital fire plan.

# 7. Distribution

Controlled master copy: office of the head of the institution, Preview Hospital, with the quality or accreditation coordinator.
Copies issued to: the laboratory; the imaging department; the named laboratory-safety lead; the person who issues personnel-monitoring devices; the role that maintains imaging signage; the infection-prevention team; the environment-and-waste function; biomedical engineering; and occupational health.
The current version is available to all staff at [Hospital to define — intranet location or nursing station folder]. The laboratory-safety programme, the spill method, the pre-imaging checklist and the current signage list — the working documents this policy requires — are held in the laboratory and in imaging.
Superseded versions are withdrawn from all points of use on issue of a revision, and one dated copy of each is retained by the quality or accreditation coordinator.

# 8. Required Records / Evidence Checklist

The records below are the documented evidence for this standard. Each record is listed against the objective element that requires it.

AAC.6.a — The laboratory-safety programme is implemented

- The written laboratory-safety programme covering biosafety at the bench (including a biosafety cabinet where work generates infectious aerosols or microbiological culture is performed, and the prohibition on mouth pipetting), chemical hygiene (labelling, separation of incompatibles, reagent safety information at the point of use, no food in a reagent refrigerator), fire as it arises from laboratory work pointing at the hospital fire plan, specimen and reagent spills at the bench other than the blood-and-body-fluid spill owned by the infection-prevention policy, bench-sharps other than bloodborne-pathogen collection sharps, and pre-treatment of microbiological waste before it enters the biomedical-waste stream owned by the environment-and-waste policy

- the named lead and the place the programme is held

- records showing the programme in use on working days — protective equipment at the bench including laboratory-specific equipment beyond standard precautions, biosafety-cabinet certification current where a cabinet is required and the cabinet not used as a shelf, labelled reagents, a replenished spill kit, pre-treatment of microbiological waste where generated, and recorded stops of work that could not be done safely

- the named lead's walk records at the stated interval

- the laboratory-safety incident register with date and time, what happened, who was involved, immediate action and the change made, covering reagent or specimen spills, bench-sharp injuries, chemical exposures, fire or near-fire at the bench, work in an uncertified cabinet, and microbiological waste that left without pre-treatment

- the review record at the stated forum and interval, and revisions of the programme after a gap

- the recorded division that bloodborne-pathogen sharps injuries during collection continue under the infection-prevention and occupational-health policies and are not duplicated here

- the audit sample at step 8 of walks that occurred and of incidents that were reviewed rather than of a folder titled laboratory safety

AAC.6.b — Laboratory personnel are appropriately trained in safe practices and are provided with appropriate safety measures

- Training records before unsupervised bench work and at the stated interval, covering biosafety, chemical hygiene, spill response, bench-sharps, laboratory fire and microbiological-waste pre-treatment, with return demonstration of the spill method and of the protective equipment the person's work uses

- records that protective equipment, the spill kit, reagent safety information and a biosafety cabinet where required were provided at the point of work and not as a personal purchase

AAC.6.c — Patients are appropriately screened for safety / risk before imaging

- The written pre-imaging checklist by modality actually provided (pregnancy where ionising radiation to abdomen or pelvis applies

- contrast history and renal risk where contrast is used

- metal and implant contraindications where MRI is provided — MRI not assumed)

- sample checklists completed before the study, against the unique identification number, with the action taken on a positive screen

- the named role that screens

AAC.6.d — Imaging personnel and patients use appropriate radiation safety and monitoring devices where applicable, and are trained in imaging safety practices and radiation-safety measures

- Personnel-monitoring issue, return and dose records as the AERB authorisation requires, at the stated interval, devices not shared

- lead aprons, thyroid shields and other protocol devices available, intact and used, with cracked devices withdrawn

- training records before unsupervised radiation work and at the stated interval

- a recorded statement that numeric dose limits are those the AERB authorisation requires and are not restated in this policy

AAC.6.e — Imaging signage is prominently displayed in all appropriate locations

- The current list of imaging-signage locations and the signs in place — radiation trefoil, restricted-area notice, pregnancy caution, and the PC-PNDT notice where ultrasonography is performed

- the named role that maintains them

- a check that this list is not the service display and not isolation signage

# 9. Revision History

| Version | Date | Description of Change |
| --- | --- | --- |
| 1.0 | 17-08-2026 | Initial release. |

# Disclaimer

This document is a template prepared for the guidance of Preview Hospital and must be reviewed, adapted and formally approved by Preview Hospital before use. Every entry marked [Hospital to define] must be replaced with the hospital's own decision; a document issued with those markers left in place is not an approved policy.
Several requirements in this document are statutory rather than advisory — in particular those arising under the Bio-Medical Waste Management Rules, 2016, the Atomic Energy Act, 1962 and the Atomic Energy (Radiation Protection) Rules together with the authorisations issued by the Atomic Energy Regulatory Board, and, where ultrasonography is performed, the Pre-Conception and Pre-Natal Diagnostic Techniques (Prohibition of Sex Selection) Act, 1994. Statutory requirements change, and State authorities impose additional or stricter conditions. Preview Hospital is responsible for verifying the current text of any rule cited here and the conditions attached to its own authorisations and licences; this document does not constitute legal advice.
The clinical and technical content reflects recognised national and international guidance current at the date of preparation. Preview Hospital remains responsible for verifying that it is current and consistent with the edition of the accreditation standard against which it is being assessed.
This document is not issued by, endorsed by, or affiliated with NABH, the World Health Organization, the National Centre for Disease Control, the Food Safety and Standards Authority of India, any Pollution Control Board, or any other body named in it. Wording is original; no text has been reproduced from the standards, rules or guidelines referenced.