Standard Operating Procedure

Fire and Non-Fire Emergencies

«Hospital Name» — SOP-FMS.5 — Version 1.0

| SOP No. | SOP-FMS.5 | Version | 1.0 |
| --- | --- | --- | --- |
| Effective Date | «________» | Review Frequency | Annual, or on change |

# 1. Purpose

This SOP tells fire wardens, facilities, clinical heads and quality staff, in one place, how this hospital detects, abates and contains fire and the non-fire emergencies it has named; how the exit plan is documented and displayed; how mock drills meet the twice-a-year floor; how fire equipment is maintained; and how essential services continue when a ward, OT or plant is lost. If you are a fire warden, hold the drill file, or run a service this hospital offers, this SOP is for you.

A smoke detector trips at 3am in a ward corridor. This SOP is what tells the night-duty lead who to notify, how the exit plan evacuates that ward's dependent patients, and which record logs the event -- not an improvised response because the plan was never actually drilled at night.

We do this because a fire or a disaster doesn't wait for daytime staffing or a convenient moment. Patients who can't walk, can't hear an alarm, or can't move themselves depend entirely on a plan that was actually tested with the people who'd really be on duty -- not one that only looks good on paper.

To define how «Hospital Name» detects, abates and contains fire and non-fire emergencies; documents and displays its exit plan; runs mock drills meeting the twice-a-year floor; maintains fire-related equipment and infrastructure; and continues essential services when a ward, OT or plant is lost.

# 2. Scope

so every area and every emergency this hospital could actually face has a named plan -- and so staff aren't improvising a response to something that was never thought through in advance.

Applies to every occupied area of «Hospital Name» and every non-fire emergency this occupancy actually faces. Does not cover laboratory bench chemical hygiene, medical-gas leak handling at source, electrical backup testing, or outsourced-service quality monitoring -- those remain their own SOPs. See Related SOPs.

# 3. Responsibility

a blocked exit, a wedged-open fire door, or a year with fewer than two drills is how a plan that looks fine on paper fails the people who actually need it. Naming who holds the plan and who executes it is what keeps that from happening.

Head of the Institution: Is accountable that fire and non-fire emergency plans exist and are provisioned as this SOP requires.

Named Fire/Facilities Lead: Holds the plans, exit display, drill records and fire-equipment maintenance.

Fire Wardens and Night-Duty Leads: Execute the exit plan.

Clinical Heads: Execute continuity for the services they run.

Quality/Accreditation Coordinator: Audits a sample of the records this SOP produces, at «Hospital to define» frequency.

All Staff: Treat a blocked exit, a discharged extinguisher left in place, a fire door wedged open, or a year with fewer than two drills, as defects to report.

# 4. What to do

## Detecting, stopping and containing fire and other emergencies

What actually detects a fire at 3am, stops it before the stair is lost, and contains it so patients in another ward can keep being cared for -- that's what this item is about. A municipal fire certificate proves a plan was once inspected; it isn't the plan itself.

We have plans and provisions for early detection, abatement and containment of fire and non-fire emergencies: detection (smoke/heat/manual call points), abatement (extinguishers, and hose reels/hydrant/sprinkler only where our occupancy requires them), containment (fire doors, shutters, compartments as built).

«Hospital to define: the fire plan (detection, abatement, containment, who is fire warden, how a dependent patient is moved), the non-fire plans for events this occupancy actually faces, and the provisions installed.» An event this occupancy doesn't face -- say, a basement flood in a single-storey building with no basement -- is a recorded absence, not a gap.

## An exit plan people can actually find and follow

A framed plan in the board room, or an exit arrow pointing at a locked grilled window, does nothing for a first-time family or a night nurse trying to get patients out safely. The plan only protects anyone if it's where people can actually see and follow it.

We document and display an exit plan: how each occupied floor leaves, the assembly point, who sweeps -- displayed where it can actually be seen.

«Hospital to define: how the exit plan is documented, where it's displayed, and how a change in a fire door or a ward move updates the display.»

## Running real mock drills, twice a year

A drill photograph of day-shift administration while the OT continues unaware doesn't prepare anyone for a real event. A drill only counts when the people who'd actually be on duty are the ones who respond -- including the night shift, and including moving a patient who can't move themselves.

We hold mock drills at least twice a year -- that interval isn't something the hospital can shorten or skip.

A drill tests detection-to-evacuation (or shelter-in-place) with the people who'd actually be on duty, including a night or weekend sample over the year, and includes a dependent-patient move.

Fire and non-fire drills don't need to be separate events eight times over -- they need to cover, across the year, fire and at least one named non-fire emergency.

«Hospital to define: how drills are scheduled so the twice-a-year floor is met, who observes, and how a failed objective is recorded and re-drilled.»

## Keeping fire equipment actually working

A plan to contain fire fails the moment the door closer is disconnected and the extinguisher gauge sits in the red. A vendor contract with no job card protects nobody -- only equipment that's actually been checked does.

We maintain fire-related equipment and infrastructure. This covers:

1. Detectors.

2. Alarms.

3. Extinguishers.

4. Hose reels.

5. Fire doors.

6. Emergency lighting.

7. Any pump or sprinkler this occupancy actually has.

«Hospital to define: which fire-related equipment this occupancy actually has, the maintenance task and interval for each, who attends, and the rule that a failed detector zone or a discharged extinguisher is replaced or isolated until restored.»

## Keeping essential services running when something's lost

"Patients will be shifted to a nearby hospital" with no named hospital and no list of who can't safely be moved isn't a plan -- it's a sentence that sounds like one, and it fails exactly when it's needed most.

We have a service continuity plan: how our essential services continue, pause, divert or move internally when a ward, OT or plant is lost.

«Hospital to define: how each essential service continues or is paused, the named receiving arrangement if diversion is the plan, how medical-gas and electrical backups are used in that hour, and who declares continuity versus full evacuation.» Services we don't offer don't need a continuity annex.

# 5. Records

This SOP produces and maintains Records 1-5 (see the FMS.5 Records folder).

# 6. Related SOPs

As-Built Drawing Set (FMS.1.b)

Monthly Facility Safety Round (FMS.2.d)

Medical Gases, Vacuum and Compressed Air SOP (FMS.4)

Laboratory Safety Programme (AAC.6)

Hospital's Service Directory (AAC.1)

# 7. Review

This SOP is reviewed at «Hospital to define the review interval», and sooner when a drill failed or a fire-door closer was found disconnected, or when a related SOP listed above is updated.

# Appendix — NABH Standard Cross-Reference and Evidence Basis

For the Quality Coordinator and NABH assessment use only. The reasoning and real-world triggers for each requirement are already in the body above -- this appendix adds only what the body deliberately leaves out: the exact objective-element code, how this SOP's scope is kept separate from neighbouring standards, and the verified technical source for anything cited.

## Detecting, stopping and containing fire and other emergencies — FMS.5.a (Core, ASTERISKED)

Boundary with other standards:

• A laboratory's own fire-related sentence in its safety programme uses this hospital plan; it doesn't replace it.

Verified source(s):

• Bureau of Indian Standards. (2016). National Building Code of India, 2016.

• National Disaster Management Authority. (2016). National Disaster Management Guidelines: Hospital Safety.

## An exit plan people can actually find and follow — FMS.5.b

## Running real mock drills, twice a year — FMS.5.c (Commitment)

Boundary with other standards:

• A drill that harmed someone is also a PSQ.5 incident.

## Keeping fire equipment actually working — FMS.5.d (Commitment, ASTERISKED)

Boundary with other standards:

• The monthly facility safety round may find a fire door or lighting failed; this plan is what repairs it -- a repair programme, not a second round.

Verified source(s):

• Bureau of Indian Standards. (2016). National Building Code of India, 2016 (IS 2190 portable-extinguisher practice).

## Keeping essential services running when something's lost — FMS.5.e (Achievement)

Boundary with other standards:

• This depends on, but is separate from, the electrical backup test (FMS.1.e) and outsourced-service quality monitoring (ROM.4.e).

Verified source(s):

• National Disaster Management Authority. (2016). National Disaster Management Guidelines: Hospital Safety.

## Full reference list

• National Accreditation Board for Hospitals and Healthcare Providers. (2022). Standards for Small Healthcare Organisations (3rd ed.). NABH SHCO Accreditation Programme page. Facility Management and Safety chapter, standard FMS.5.

• Bureau of Indian Standards. (2016). National Building Code of India, 2016. Fire and life-safety framework as the local fire authority has applied it to this occupancy; not a universal sprinkler or occupancy-subdivision mandate. IS 2190 is NBC-pointed portable-extinguisher practice, not an extra statute.

• National Disaster Management Authority. (2016). National Disaster Management Guidelines: Hospital Safety. Non-fire and hospital-continuity framework, not an Act of Parliament.

• World Health Organization. (2015). Hospital Safety Index: Guide for Evaluators (2nd ed.). Evaluator framework, not a mandated score.