Standard Operating Procedure

Infection Prevention and Control in Support Services

«Hospital Name» — SOP-HIC.3 — Version 1.0

| SOP No. | SOP-HIC.3 | Version | 1.0 |
| --- | --- | --- | --- |
| Effective Date | «________» | Review Frequency | Annual, or on change |

# 1. Purpose

This SOP sets out how «Hospital Name» controls infection risk in the support services that keep the hospital running: building engineering controls like ventilation and water quality, infection-control planning before any construction or renovation, housekeeping, biomedical-waste handling, laundry and linen, and kitchen sanitation and food handling.

A renovation starts next to a ward with no dust barrier, or a sharps box is left overflowing in a corridor -- neither looks like a clinical failure, but both can put an infection straight into a patient who never went near the construction site or the waste bin. This SOP is what makes support-service areas as accountable for infection control as the wards are.

We do this because infection does not only travel through clinical contact -- contaminated water, dust from a renovation, poorly handled waste, improperly washed linen, or food held at the wrong temperature can each put a patient at risk just as directly.

To define how «Hospital Name» implements infection prevention and control in support services: engineering controls, infection risk during construction and renovation, housekeeping, biomedical-waste management, laundry and linen management, and kitchen sanitation and food handling.

# 2. Scope

so every support department -- maintenance, housekeeping, laundry, kitchen, the biomedical-waste handler -- knows its own work is part of infection control, not separate from it.

Applies to all support-service areas of «Hospital Name»: maintenance and engineering, housekeeping, biomedical-waste handling, laundry, kitchen and dietetics, along with the Infection Control Officer and Nurse who oversee them. It covers the infection-control requirements for how each of these services is run. It does not cover the facilities-engineering infrastructure itself, clinical precautions used in direct patient care, or prevention of specific device- and procedure-linked infections -- each of those is set out in its own separate SOP in this chapter or in the facilities-management chapter.

# 3. Responsibility

support-service infection control that depends on "someone will notice" rather than a named owner for ventilation, waste, linen and food is exactly how a contamination risk goes unnoticed until a patient is affected.

Infection Control Officer: Specifies engineering-control requirements; completes the infection-control risk assessment before construction/renovation; approves the biomedical-waste and laundry protocols.

Maintenance / Facilities Team: Implements and maintains engineering controls; complies with infection-control risk-assessment precautions during projects.

Housekeeping In-Charge: Implements the area-wise cleaning schedule; maintains colour-coded equipment allocation.

Biomedical-Waste Handler / Coordinator: Manages day-to-day segregation, storage and handover of biomedical waste; all staff segregate waste at the point it is generated.

Laundry In-Charge: Runs the laundry and linen process per the thermal/chemical disinfection protocol.

Kitchen / Dietetics In-Charge: Implements kitchen sanitation and food-handling practice.

Medical Superintendent: Approves infection-control risk assessments before construction begins; oversees medical-fitness clearance for food handlers.

# 4. What to do

## Maintain engineering controls that prevent infection

Air that does not change often enough in an operating theatre, or water that is never tested, can carry infection to a patient with nobody in the room doing anything wrong at all -- the building itself has to be part of infection control.

«Hospital Name» maintains appropriate engineering controls, including:

1. Adequate ventilation and air changes in clinical areas, following national or ASHRAE/ISHRAE guidance -- «minimum 6 air changes per hour in general wards, 12 in the operating theatre, negative pressure in isolation rooms».

2. HEPA filtration where required (operating theatre, isolation areas).

3. Potable water quality monitoring and storage-tank cleaning «quarterly».

4. Sewage treatment and effluent management, so the hospital does not contaminate its own surroundings.

The Infection Control Officer and the maintenance team conduct a joint round «monthly» to verify these controls are actually functioning. Any deficiency found is reported for correction and tracked through to closure.

## Assess and control infection risk before any construction or renovation

Dust and vibration from a renovation can carry fungal spores straight into a ward's air supply -- the risk has to be assessed and controlled before the first wall comes down, not noticed afterward.

Before any construction or renovation in or next to a patient-care area, the Infection Control Officer completes an infection-control risk assessment (ICRA). The ICRA classifies the project type and the patient-risk group of the area affected, to decide what precautions are required:

1. Dust barriers, negative pressure in the construction zone, sealed windows.

2. A separate traffic route for construction workers and debris, away from clinical areas.

3. Air monitoring during and after the project.

4. Terminal cleaning of the area before it is returned to clinical use.

The ICRA and its mitigation plan are approved by the «Medical Superintendent» before work begins. Compliance is monitored by the Infection Control Nurse for as long as the project runs.

## Follow housekeeping procedures that actually prevent cross-contamination

A mop used in an isolation room and then carried straight into the next ward carries the infection with it -- the colour-coding and separate schedules exist because "clean enough" cleaning still spreads germs.

«Hospital Name» follows housekeeping procedures that prevent cross-contamination between areas:

1. A cleaning schedule defines the frequency for every area, with high-risk areas cleaned «three times a day plus terminal cleaning after every discharge».

2. Colour-coded cleaning equipment -- mops, buckets, cloths -- stops equipment moving between areas and spreading contamination.

3. Disinfectant choice and dilution follow the list the Infection Control Committee has approved.

4. A spill-management protocol: contain, disinfect, clean, dispose.

5. Terminal cleaning follows every discharge from isolation or barrier nursing.

6. Housekeeping staff are trained in infection-control cleaning methods at induction and «annually».

How effective this actually is gets checked separately, as part of this hospital's infection-control surveillance.

## Handle biomedical waste safely, from the point it is generated to final disposal

Mixing a used needle in with ordinary waste, or letting waste sit unsegregated past its safe storage window, turns an everyday bin into a sharps or infection hazard for the next person who handles it -- often someone with no idea what is inside.

Biomedical waste at «Hospital Name» is segregated at the point it is generated into colour-coded categories, following the Bio-Medical Waste Management Rules, 2016:

1. Yellow: human anatomical waste, soiled waste, expired medicines, and chemical (cytotoxic) waste.

2. Red: contaminated recyclable waste -- tubing, syringes without needles, gloves.

3. White, puncture-proof: sharps -- needles, blades, glass.

4. Blue: glassware waste, where applicable.

Waste is stored in a designated, secured intermediate storage area for no longer than «48 hours». Collection, transport, treatment and disposal are carried out by the «authorised common biomedical waste treatment facility» under a valid agreement. Daily segregation logs, transport manifests, monthly returns and the annual report to the State Pollution Control Board are all kept on record.

## Manage laundry and linen so washing itself does not spread infection

Soiled linen carried through a clean corridor, or washed without reaching a disinfecting temperature, can move an infection from one patient's bed to the next patient who gets that linen.

«Hospital Name» follows laundry and linen management that prevents infection transmission:

1. Soiled linen is collected in leak-proof bags or containers at the point it is used; colour coding keeps infectious linen separate from routine soiled linen.

2. Transport to the laundry uses covered trolleys on a defined route that avoids clean areas.

3. Washing uses thermal disinfection -- «65°C for 10 minutes or 71°C for 3 minutes» -- or chemical disinfection for heat-sensitive items.

4. Clean linen is stored in a covered, designated area, kept separate from soiled holding.

5. A linen inventory and replacement schedule keep any visibly damaged or stained linen out of circulation.

The laundry area's layout follows a one-way dirty-to-clean flow, and staff handling infectious linen wear PPE -- heavy-duty gloves, gown, mask.

## Keep kitchen sanitation and food handling safe enough to prevent foodborne infection

A patient already fighting illness is far more vulnerable to foodborne infection than a healthy person would be -- the kitchen is feeding people with the least margin for error.

«Hospital Name» follows kitchen sanitation and food-handling practice that prevents foodborne infection:

1. Kitchen staff undergo medical-fitness screening -- stool culture and skin examination -- before employment and «annually» after.

2. Personal hygiene: hand washing before handling food, hair covering, a clean uniform, and no jewellery in food-preparation areas.

3. Food storage keeps the cold chain intact -- «below 5°C for perishables» -- with dry stores kept above floor level and rotated first-in-first-out.

4. Cooking brings the core temperature of food to «74°C».

5. Food is served within «2 hours» of cooking, or kept hot at or above 63°C; therapeutic diets are labelled and dispatched per the dietitian's order.

6. Kitchen surfaces and equipment are sanitised «after every meal service»; pest control is carried out «monthly».

7. Water used in the kitchen is tested for potability «quarterly».

# 5. Records

This SOP produces and maintains Records 1-6 (see the HIC.3 Records folder).

# 6. Related SOPs

Hospital's Hospital Infection Prevention and Control Programme SOP (HIC.1)

Hospital's Infection Prevention and Control Surveillance SOP (HIC.5)

Hospital's facilities-management engineering-controls SOP (FMS)

# 7. Review

This SOP is reviewed annually, or sooner if a biomedical-waste segregation breach, a construction-related infection-control deviation, or a food-safety non-conformance is identified, or if a related policy named above in this chapter is revised.

# Appendix — NABH Standard Cross-Reference and Evidence Basis

For the Quality Coordinator and NABH assessment use only. The reasoning and real-world triggers for each requirement are already in the body above -- this appendix adds only what the body deliberately leaves out: the exact objective-element code, how this SOP's scope is kept separate from neighbouring standards, and the verified technical source for anything cited.

## Maintain engineering controls that prevent infection — HIC.3.a

## Assess and control infection risk before any construction or renovation — HIC.3.b

## Follow housekeeping procedures that actually prevent cross-contamination — HIC.3.c

Boundary with other standards:

• How housekeeping effectiveness is objectively monitored (visual audits, ATP/microbiological testing) is owned by HIC.5.e; this step is the procedure itself.

## Handle biomedical waste safely, from the point it is generated to final disposal — HIC.3.d

Verified source(s):

• Central Pollution Control Board, Ministry of Environment, Forest and Climate Change, Government of India. (2016). Bio-Medical Waste Management Rules, 2016, as amended. New Delhi: CPCB -- the legally binding colour-coded segregation and disposal scheme this hospital's biomedical-waste handling is built on.

## Manage laundry and linen so washing itself does not spread infection — HIC.3.e

## Keep kitchen sanitation and food handling safe enough to prevent foodborne infection — HIC.3.f

## Full reference list

• National Accreditation Board for Hospitals and Healthcare Providers. (2022). Standards for Small Healthcare Organisations (3rd ed.). NABH SHCO Accreditation Programme page. Hospital Infection Control chapter, standard HIC.3.

• Central Pollution Control Board, Ministry of Environment, Forest and Climate Change, Government of India. (2016). Bio-Medical Waste Management Rules, 2016, as amended. New Delhi: CPCB -- the legally binding colour-coded segregation and disposal scheme this hospital's biomedical-waste handling is built on.