Standard Operating Procedure

Proactive Patient Safety Risk Analysis

«Hospital Name» — Supports PSQ.1.e

| SOP No. | [Hospital to define] | Version | 1.0 |
| --- | --- | --- | --- |
| Effective Date | ________ | Review Frequency | Annual, or on change |
| Prepared By Quality Coordinator Name: ________ Signature: ________ |  |  |  |
| Approved By Medical Superintendent Name: ________ Signature: ________ |  |  |  |

# 1. Purpose

This SOP describes how «Hospital Name» carries out proactive analysis of patient safety risks — before an incident happens — and turns the findings into real improvements.

# 2. Scope

Applies to at least one patient safety risk analysed proactively every year, covering both clinical and non-clinical processes and areas.

# 3. Responsibility

Safety Committee: selects the risk/process to analyse, runs the analysis, and tracks resulting improvements.

Patient Safety Officer: coordinates the analysis and reports outcomes to the Safety Committee.

# 4. Procedure

1. Select a process or risk for proactive analysis — prioritise a high-risk, high-volume, or error-prone process (e.g. IV pump programming, medication look-alike risk, handoff between shifts).

2. Choose an analysis tool suited to the question:

— Failure Mode and Effects Analysis (FMEA) examines a process step by step for every way it could fail, then prioritises the failure modes with the greatest potential impact. Use it when you want to redesign a specific process before it fails.

— Hazard Identification and Risk Analysis (HIRA) identifies hazards and rates each by likelihood and severity, so the highest-priority risks get attention first. Use it for a broader area (e.g. an entire ward) rather than one process.

— Hazard Vulnerability Analysis (HVA) identifies the emergencies the hospital could face and their direct and indirect effect on operations. Use it for disaster/business-continuity style risks, not routine clinical processes.

3. Worked example (FMEA on IV pump programming): list every step of programming an infusion pump; for each step, list ways it could fail (e.g. wrong rate entered, wrong drug library selected); score each failure mode for severity, likelihood of occurrence, and likelihood of detection; multiply the three scores to get a Risk Priority Number; redesign the steps with the highest numbers first (e.g. add a mandatory second-person check for high-risk drug libraries).

4. Document the tool used, the findings, and the risk ranking in the Proactive Risk Analysis Record (Record 7).

5. Implement improvements based on the findings — do not simply document the risk and stop there.

6. Confirm at least one such analysis has been completed in the current year before the annual programme review.

# 5. Records

Supported by the Proactive Risk Analysis Record (Record 7) of the PSQ.1 Required Records pack.

# 6. Related Documents

Policy on Patient Safety Programme (HCO/PSQ/POL/01) — PSQ.1 policy, Section 3.5 (proactive risk analysis)

# 7. Review

Reviewed annually, or immediately if a proactive analysis reveals a gap in this procedure.

# 8. References

Institute for Healthcare Improvement. (n.d.). Failure Modes and Effects Analysis (FMEA) Tool. https://www.ihi.org/library/tools/failure-modes-and-effects-analysis-fmea-tool

California Hospital Association. (n.d.). Hazard Vulnerability Analysis. https://calhospitalprepare.org/hazard-vulnerability-analysis